PUDUCHERRY TECHNOLOGICAL UNIVERSITY

Department of Electronics & Communication Engineering

Resource-Aware FPGA-Based CNN Accelerator for Edge AI

A Critical Literature Survey, Comparative Analysis and Research-Gap Study

Mini Project — Literature Survey Stage

Team: Yashaswini G M (2301116051) · Dejaaswini E (2301109022) · Imayavaraman L (2301109038) · Indugula Surya Teja (2301109039)

Guide: Dr. S. Tamilselvan, Professor, Dept. of ECE

Survey compiled: September 2026

How to read this document. Every quantitative claim is tied to a numbered source in the reference list. NR means the value was not reported in the text retrieved for this survey (it may exist in a figure or table that could not be opened). Unverified means the item was seen only through a secondary citation or a title. Items marked (datasheet) or (illustrative) come from the authors’ device knowledge or from a worked example rather than from the literature, and must be re-checked before use in a report.

# Abstract

Deploying convolutional neural network (CNN) inference on low-cost FPGAs requires balancing classification accuracy, latency, throughput, resource utilisation, timing and power under a very small device budget. This survey reviews the FPGA CNN-accelerator literature that is closest to a planned project: a custom-Verilog, INT8-quantised CNN for MNIST on an Artix-7 XC7A35T (Basys 3), with configurable multiply-accumulate (MAC) parallelism and evaluation by LUT, FF, DSP, BRAM, timing, latency, throughput and power.

About thirty sources (2015–2026) were screened and twenty-one were analysed in depth across lightweight networks, quantisation, accelerator architecture, memory and dataflow, parallelism scaling and design-space exploration (DSE), power and energy, RTL-versus-HLS design flow, and verification. Three findings dominate. First, INT8 and fixed-point LeNet-class accelerators on Artix-7 are established; a 2026 study already presents a training-to-RTL INT8 pipeline on the same die as the Basys 3 [1]. Second, parallelism scaling has been demonstrated in hand-written Verilog [3], on Arty-7 with power [4], through FINN folding [7] and in PE-array scaling on a large device [9], but no verified source reports a controlled MAC-lane sweep on an XC7A35T-class device that combines all resource and timing metrics with credible power and per-configuration bit-exact verification. Third, power is frequently omitted [2] or only estimated [5][11], and estimation mode alone can change energy per inference by nearly a factor of two [26].

Seven research gaps are derived with evidence, and a defensible contribution is proposed: a parameterised RTL accelerator whose parallelism, multiplier mapping and dataflow are controlled by parameters, evaluated with a validated analytical model, disclosed power methodology and configuration-wise bit-exact equivalence.

Keywords: FPGA, CNN accelerator, INT8 quantisation, Artix-7, custom RTL, MAC parallelism, design-space exploration, energy efficiency, edge AI.

## Contents

1  Introduction and survey methodology

2  Technical background

3  Literature review by theme (3.1–3.9)

4  Comparison tables (Tables 1–9)

5  Critical cross-paper analysis and design implications

6  Research gaps (Gap 1 – Gap 7)

7  Research-gap matrix

8  Novelty audit and the critical question

9  Final research-gap statements (three versions)

10  Recommended contribution and experimental design

11  Limitations of this survey · References · Appendices

# 1  Introduction and survey methodology

## 1.1  Motivation and project scope

Edge-AI image inference needs low latency and low power, but CNN deployment on low-cost FPGAs is limited by scarce logic, DSP, block RAM and power headroom. The project reviewed here proposes a resource-aware accelerator with this pipeline: MNIST → lightweight two-layer CNN trained in PyTorch → INT8 post-training quantisation → weight extraction → custom Verilog RTL (convolution, ReLU, pooling, fully connected, on-chip BRAM, parallel MAC units) → verification against a Python golden model → Vivado implementation on the Basys 3 (Artix-7) → evaluation of latency, throughput, LUT/FF/DSP/BRAM, timing and power for 1, 4 and 8 MAC configurations.

## 1.2  Research question

How can CNN inference be efficiently mapped onto a resource-constrained FPGA, and how does changing the degree of hardware parallelism affect the trade-off between classification accuracy, latency, throughput, resource utilisation, timing, power and energy efficiency?

## 1.3  Survey methodology

Search strategy. Query families covered exact project similarity (e.g. INT8 CNN accelerator FPGA Artix-7; Verilog CNN accelerator Basys 3), architecture (parallel MAC/PE, convolution engines), parallelism and DSE, memory and dataflow, power and energy, RTL versus HLS, and verification (golden model, bit-exact). Forward and backward citation chasing was applied to the strongest papers (for example the reference lists of [2] and [3]). The query log is in Appendix A.

Inclusion criteria. A work was retained only if it implements or rigorously analyses CNN (or closely related NN) inference on an FPGA and is close to the project on at least three of: low-cost or Xilinx device, integer/fixed-point precision ≤16 bit, custom RTL, explicit PE/MAC organisation, multi-configuration or DSE analysis, resource and timing reporting, power or energy analysis, verification method. Generic surveys were used only for background.

Depth of reading. Three papers were read in substantial part [1][2][3]. The remainder were analysed from abstracts, results tables and text excerpts retrieved during searching; where a table or figure could not be opened the value is recorded as NR. Sources behind paywalls (notably most IEEE Xplore and ACM full texts) were not accessible, so IEEE venues such as FPL, FCCM, TCAS-I and TVLSI are under-represented. This is the main limitation of the survey (Section 11).

Similarity score (0–10). A transparent rubric is used: FPGA CNN accelerator (2); low-cost/Xilinx FPGA (2); INT8/fixed-point (2); custom RTL (1); parallel MAC/PE architecture (1); resource/performance analysis (1); multiple hardware configurations or DSE (1). Conventions: Xilinx-but-not-low-cost = 1 of 2; Intel low-cost = 1 of 2; 9–16-bit fixed-point = 1 of 2; RTL/HLS not reported = 0. Half-points are allowed. Scores measure similarity to this project, not paper quality.

## 1.4  Organisation

Section 2 gives the technical background. Section 3 reviews the literature by theme. Section 4 presents comparison tables. Section 5 analyses the evidence across papers. Sections 6–9 develop the research gaps, gap matrix, novelty audit and gap statements. Section 10 turns the gaps into an experimental plan.

# 2  Technical background

## 2.1  CNN inference workload

A convolutional layer performs H_out × W_out × C_out × C_in × K² MAC operations; a fully connected (FC) layer performs N_in × N_out MACs. Pooling and activation add comparison logic but negligible arithmetic. For small networks such as LeNet-5 [31] the inference workload is therefore a modest, regular loop nest, and hardware speed is governed by how many of these MACs can be issued per clock and how quickly operands can be fed from on-chip memory [3][13][14].

## 2.2  The target device: Artix-7 XC7A35T (Basys 3)

The Basys 3 board carries an XC7A35T-1CPG236C device (Digilent specification). Typical datasheet resources are 20,800 LUTs, 41,600 flip-flops, 90 DSP48E1 slices and 50 × 36 Kb block RAMs (1,800 Kb in total) (datasheet — verify against Xilinx DS180). The Cmod A7-35T used by [1] contains the same die family member, so results on it are directly comparable in resource terms. By contrast the XC7A200T used by [2] has roughly 6.5 times as many LUTs (datasheet), so absolute utilisation numbers from that paper cannot be transferred to the Basys 3.

## 2.3  INT8 post-training quantisation

Symmetric per-tensor INT8 PTQ maps a real tensor t to q = clip(round(t/s), −128, 127) with s = max|t|/127; MACs accumulate in INT32 and each layer is re-quantised with a shift and saturation before the next layer [1]. Because the integer arithmetic is fully specified, a bit-accurate software reference can be built and compared with the RTL layer by layer [1]. A design-level consequence, examined in Section 5, is that classification accuracy does not depend on the parallelism degree if every configuration implements the same integer arithmetic.

## 2.4  Parallelism, dataflow and memory

Mao et al. distinguish four sources of parallelism in a CNN layer: multiplications inside one kernel window, parallel input feature maps (PM), parallel output feature maps (PN) and, for FC layers, parallel output neurons (PFO) [3]. Dataflows are classified by what stays stationary in a PE — weights, outputs, inputs or rows — and determine on-chip reuse and memory traffic [15][29]. Line buffers create sliding windows so that each input pixel is read once per layer [1][8][11]. Loop tiling, interchange and unrolling define the design space explored analytically in [13][14][15].

## 2.5  Design flow: custom RTL versus HLS

HLS tools (Vitis HLS, FINN, hls4ml) raise productivity and ease exploration, while hand-written RTL offers direct control of timing and structure [22][23][24]. The trade-off is quantified in Section 3.8.

# 3  Literature review by theme

## 3.1  Foundations: analytical design-space exploration

Zhang et al. framed FPGA CNN design as a roofline problem: for a given loop tiling and unrolling, compute throughput and off-chip bandwidth are quantified so that under-used logic or bandwidth can be identified [14]. Ma et al. extended this by optimising loop operation and dataflow together [15]. Both target large devices and large networks, but they established the vocabulary — unroll factors, tiling and bandwidth — used by every later parallelism study. Recent surveys [35][36] and a state-of-the-art review [34] describe the same building blocks: PEs with local memory and MAC units, tiling to fit BRAM, pipelining and double buffering. Foundational architectures such as Eyeriss and the TPU are cited in [1] and are listed in the reference appendix as foundational items to be checked.

## 3.2  Lightweight CNNs on small FPGAs

MNIST-class LeNet variants dominate the low-cost literature. Mao et al. implement LeNet-5 with hand-written Verilog on a Zynq-7020, reaching 248 µs per image with a 16-bit fixed-point format that cost 0.26% accuracy relative to FP32 [3]. Cruz et al. deploy a single-convolution MNIST network (20 kernels of 9×9 followed by 2000→100→10 FC layers) on an Artix-7 XC7A200T via Matlab-to-Vitis-HLS, obtaining 96.40% accuracy on a 2,000-image subset (95.80% on the full test set) at 5.35 ms latency [2]. Ghani et al. scale a LeNet-inspired seven-layer CNN to fit the Basys 3 for 40×40 medical images with about 91% accuracy [6]. An LPR (licence-plate recognition) accelerator replaces LeNet-5’s FC layers with convolutions so that everything stays on chip, and is ported between Intel and AMD devices from identical VHDL [8]. An approximate-MAC fixed-point HLS design cut DSP use by 78% relative to general fixed-point designs and memory by 66% relative to floating point for LeNet-5 [18], and a related Zynq design combines shortened fixed-point words, approximate MACs and loop unrolling [19]. Beyond MNIST, MobileNetV2 has been mapped at 4-bit precision to an XC7Z020 at 3.07 FPS [10], and depthwise-separable accelerators target “low-end” FPGAs [27].

Analysis. The surveyed networks range from about 60 thousand to over 200 thousand parameters, and the choice is rarely justified against a device resource model. Cruz’s 200,000-weight FC1 layer [2] is the clearest example: it contributes 37% of inference cycles and receives no speed-up from HLS directives (Section 5.1). For an XC7A35T, storage — not arithmetic — is the first constraint (Section 5.4).

## 3.3  Quantisation and accuracy-versus-hardware cost

Shanker and Teo present a symmetric per-tensor INT8 PTQ with INT32 accumulation and per-layer requantisation, and study INT4/6/8 accuracy, per-tensor versus per-channel scaling, calibration sensitivity and noise robustness [1]. Bel Haj Salah et al. show a TensorFlow-Lite-style FP32-to-INT8 PTQ framework with a 94% power-delay-product reduction and large LUT/DSP efficiency gains for ResNet50 [16]. Cruz et al. select per-layer (W, I) fixed-point formats by range coverage and sensitivity analysis: at the convolution output a 4-bit word covers only 69% of the dynamic range with a 36.21% normalised error, while 8 bits gives 94% coverage and 7.81% error; saturation is preferred over wrapping, and softmax is kept in floating point [2]. Tasci et al. explore W1A1, W1A2 and W2A2 quantisation combined with folding across twelve FINN accelerators [7], and MaxNet uses Q0.8/Q1.7 8-bit arithmetic in a two-layer CNN (77% on CIFAR-10) [17].

Analysis. Accuracy loss at 8 bits is small and well documented. What is under-documented is the hardware side effect of arithmetic choices. Cruz observed that fixed-point HLS raised DSP use (14% to 18%) because Vivado preferentially inferred DSP multipliers when the schedule replicated MACs for speed [2], whereas the approximate-MAC design deliberately moved multiplications into logic to save DSPs [18]. Neither study isolates DSP-versus-LUT multiplier mapping as an explicit experimental variable.

## 3.4  Accelerator architecture

Mao et al. build a parameterised module library (convolution, pooling, ReLU, FC, control) in Verilog. A convolution block uses two DSPs, layers are pipelined with handshakes, weights are interleaved across BRAMs, and a search algorithm allocates DSPs and memory per layer [3]. A flexible accelerator built from cores containing one-dimensional PE arrays scales an int8 virtual PE from 16 to 324 instances on a VC709 [9]. A Zynq-7020 ship-recognition design uses streaming line-buffered convolution with a MAC tree and is LUT/routing-limited at 84.65% LUT utilisation [11]. Hernández Morales et al. compare MAC-based systolic arrays of 4–8-bit weights with a multiplier-less shift-and-add PE design on an Arty A7-100T [5]. Zhao et al. propose a multi-computing-engine streaming accelerator with balanced dataflow for lightweight CNNs [28]. Shanker and Teo map LeNet-5 to a row-stationary, line-buffered Verilog datapath on an XC7A35T [1].

Analysis. [3] and [1] are the closest to the proposed architecture. [3] contributes the parallelism formalism, [1] the device, precision and verification. [11] is an important warning: on a small device the parallelism ceiling can come from routing and LUT pressure rather than DSP count.

## 3.5  Memory architecture and dataflow

The LPR accelerator adopts a weight-stationary dataflow with on-chip line buffers and feature-map buffers dimensioned for stall-free streaming, and keeps all layers BRAM-resident [8]. ADS-CNN combines a time-division-multiplexed compute engine, a configurable convolution controller and per-layer on-chip buffers on a Cyclone V SoC, gaining up to 1.29× speed with 23.7% dataflow compression against tiling [12]. The XC7Z020 low-cost accelerator schedules DRAM requests with a scoreboard, reaching roughly six-fold fewer DSPs and nine-fold better BRAM efficiency than prior work [10]. Shanker and Teo include a dataflow trade-off of BRAM versus cycles per image [1]. A dataflow survey [29] and an open-source improved row-stationary accelerator [30] give taxonomies, but no FPGA study I could verify compares weight-, output- and row-stationary implementations of the same network on an Artix-7.

## 3.6  Parallelism scaling and design-space exploration

Parallelism has been studied in five distinct ways. (a) Per-layer search under DSP and memory limits [3]. (b) “Inner-parallel” optimisation on an Arty-7, where the optimised CNN cut latency by about 60% at under 2% additional power [4]. (c) FINN folding combined with quantisation on a PYNQ-Z1 [7]. (d) PE-count scaling of an int8 array, where utilisation rises from 6% to 69% of LUTs, 5% to 54% of FFs, 5% to 26% of BRAM and 1% to 18% of DSPs between 16 and 324 PEs [9]. (e) HLS directives (unroll, pipeline, dataflow) that improved latency from 9.19 to 5.35 ms, a 1.71× gain, with the FC1 loop unchanged at 1.00× [2]. DSE frameworks include loop-optimisation design spaces [13], the OpenCL Systolic-CNN parameters vec_fac, pe_num and reuse_fac [20], and accuracy–latency Pareto search with analytical latency and resource models [5].

Analysis. Each study covers a subset of the required evidence: [3] has no reported power; [4] is a non-image task with precision NR; [7] is ≤2-bit and HLS-based; [9] uses a large device; [2] compares only two design points. The published trend is unsurprising (more parallelism, lower latency, more area). What is unknown is where the knee lies on a 90-DSP device and how LUT/routing/BRAM banking, rather than DSPs, set the ceiling.

## 3.7  Power and energy

The LPR accelerator reports 1.62 mJ per inference on Zynq UltraScale+ and 3.32 mJ on Cyclone V [8]. Lee et al. report power alongside latency on Arty-7 and VC709, including an 85.49% latency and 86.91% power reduction versus a GPU for the optimised CNN [4]. Hernández Morales et al. give estimated power and energy (for the 8-bit systolic array on DS-CNN: 1,097 mW and 33.77 µJ) [5]. MaxNet reports 1.2 W using Quartus power tools and compares with CPU and GPU baselines [17]. The ship-recognition accelerator reports a Vivado post-implementation estimate of about 2.04 W [11]. An RTL clock-gating and sparsity-aware MAC suppression study reduced total power from 3.644 W to 2.442 W (33%) while LUT use rose from 44,392 to 102,580 [21]. Cruz et al. explicitly report no board-level power or energy [2].

Analysis. The methodological weak point is estimation. A verified SNN accelerator on Artix-7 reports 0.053 mJ per inference with vectorless estimation but 0.028 mJ with SAIF-based activity, a difference of almost two [26]. Although that work is not a CNN, it demonstrates that the estimation mode alone can change energy by nearly 2×, so any CNN comparison must state and control the mode.

## 3.8  Custom RTL versus HLS

Evidence from FINN indicates that an RTL matrix-vector unit yields significantly smaller circuits than HLS for small designs, HLS consumes more FFs and BRAMs, RTL has a much shorter critical path, and HLS synthesis takes at least ten times longer; LUT counts converge, with RTL up to about 15% higher for large designs [22]. On MLPerf Tiny, an hls4ml design used 58% fewer BRAMs than a FINN design but had 18.2× its latency, showing that HLS flows embody different resource–latency choices [23]. A recent hls4ml perspective reports HLS results ranging from better than hand-mapped designs to up to 2× worse [24]. Mao et al. report fewer LUTs and FFs for hand-written Verilog than for an HLS LeNet-5 on the same Zynq-7020 part [3], and Cruz et al. note that post-synthesis estimates can drift from design goals and that HLS limits low-level timing control [2]. An HLS multi-threaded-C flow stresses that producing hand-written RTL for every architecture variant is expensive [25].

Analysis. The evidence favours RTL for small, structured designs where timing and structure matter, and favours HLS for exploring many variants quickly. A parameterised RTL generator combines both benefits, which is the position the proposed project can occupy.

## 3.9  Verification and reproducibility

Shanker and Teo define an explicit integer arithmetic contract mirrored in software and RTL, perform interface-level parity checks, compare integer outputs within one LSB (with saturation and rounding rules fixed), localise mismatches by dumping intermediate nodes, and report zero mismatches across large test sets [1]. Cruz et al. use a four-stage protocol (Matlab float, Matlab fixed-point, HLS C simulation, C/RTL co-simulation) with checkpoints at the convolution output and logits, and release source code [2]. Many other verified-looking claims were found only in unreviewed open-source repositories and were excluded (Appendix B).

Analysis. Rigorous bit-exact verification exists for single design points. No source I verified demonstrates equivalence across a family of parallelism configurations, although this is where subtle bugs (banking, accumulation order, pipeline flush) arise.

# 4  Comparison tables

All tables use the source numbers exactly as retrieved. NR = not reported in retrieved text. “est.” = tool estimate rather than measurement. Rows are ordered by relevance to the project, not by year.

Table 1A. Design attributes of the closest papers

| Ref | Year / venue | FPGA | Network / dataset | Precision | Flow | Parallelism / PE / MAC | Dataflow / memory |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [1] Shanker & Teo | 2026, Electronics | Artix-7 XC7A35T (Cmod A7) | LeNet-5 (sigmoid), XOR CNN; MNIST | INT8 PTQ, INT32 acc. | Verilog RTL | INT8 MAC units; count NR | Row-stationary; line buffers; BRAM holds buffers, weight tiles, partial sums |
| [3] Mao et al. | 2023, Electronics | Zynq-7020 | LeNet-5; MNIST | 16-bit fixed | Hand-written Verilog | PM, PN, k, PFO; 2 DSP per conv block; search algorithm | Layer-pipelined with handshakes; interleaved BRAM weights; all on-chip |
| [5] Hernández Morales et al. | 2026, arXiv preprint | Arty A7-100T (Artix-7) | MLPerf Tiny (DS-CNN, ResNet) | 4–8-bit weights; Po2 | NR | MAC systolic arrays vs shift-add PEs | Systolic; BRAM |
| [2] Cruz et al. | 2026, Array | Artix-7 XC7A200T-2 | Custom MNIST CNN (20 kernels 9×9; FC 2000→100→10) | Mixed fixed-point; softmax float | Matlab → Vitis HLS | HLS unroll / pipeline / dataflow | HLS dataflow; BRAM |
| [4] Lee et al. | 2020, Opt. Express | Arty-7 and VC709 | CNN and BCNN, mm-wave RoF decisions | NR (BCNN binary) | NR | “Inner-parallel” optimisation (CNN1→CNN3) | NR |
| [6] Ghani et al. | 2024, IoT | Basys3 (Artix-7) | LeNet-inspired 7-layer CNN; 40×40 medical images | 8-bit integer | NR | Parallel processing emphasised; PE count NR | NR |
| [7] Tasci et al. | 2025, Appl. Sci. | PYNQ-Z1 (Zynq-7020) | MLP and LeNet; FashionMNIST | W1A1, W1A2, W2A2 | FINN (HLS) | Folding levels; 12 accelerators | Streaming |
| [8] LPR accelerator | 2026, Integration | Cyclone V; Zynq UltraScale+ | LeNet-5-derived, 4 conv; licence-plate OCR (36 classes) | 8-bit | VHDL | One MAC per DSP block; count NR | Weight-stationary; line buffers; BRAM-resident |
| [9] Flexible accelerator | arXiv 1912.07284 | Virtex-7 VC709 | Various CNNs | int8 vPE (and fp32 PE) | Verilog | 1-D PE array cores; 16–324 PEs | Reuse-oriented; BRAM + DRAM |
| [10] Low-cost edge accelerator | 2022, ACM (venue to confirm) | XC7Z020 (Artix-7 fabric) | MobileNetV2 (depthwise-separable) | 4-bit | NR | 220 DSP multipliers in parallel; channel parallelism | Scoreboard-scheduled DRAM; BRAM |
| [11] Ship recognition | 2026, Sensors | Zynq-7020 | AlexNet, ResNet-18 class models | NR | NR | MAC tree; per-output-channel engines | Streaming; line buffers; AXI-Stream |

Table 1A note: [1] and [3] are the closest architectures; [5] and [4] are the closest experimental methods; [6] shares the same board.

Table 1B. Reported results of the closest papers

| Ref | LUT | FF | DSP | BRAM | Freq | Latency | Throughput | Power / energy | Accuracy | Score |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [1] | NR (figure) | NR | NR | NR | 100 MHz | ms-scale | Reported; value NR | NR | FP32 vs INT8 compared; zero RTL mismatches | 8.5 |
| [3] | NR (Table 3 not opened) | NR | 96 in layer 2 (2 × PM × PN) | NR | NR | 248 µs / image | 18.5× FPS of a cited baseline | NR | 0.26% drop vs FP32 at 16-bit | 9 |
| [5] (8-bit SA, DS-CNN) | 61,612 (97%) | 22,927 (18%) | NR | 7.5 × 36 Kb (6%) | 114 MHz | 30.79 µs | 89 GOPS | 1,097 mW est.; 33.77 µJ est. | 91.95% top-1 | 9 |
| [2] | 6,340 (text); 6,324 (Table 11) | 1,268 | 43 | 4 | 100 MHz | 5.35 ms | 187 FPS | Not reported | 96.40% (2k); 95.80% (10k) | 6.5 |
| [4] | NR | NR | NR | NR | NR | ≈ −60% (CNN3 vs CNN1, Arty-7) | NR | < +2% power | NR | 7 |
| [6] | NR | NR | NR | NR | NR | NR | NR | 1.33% of K80 GPU power | ≈ 91% | 7 |
| [7] | NR | NR | NR | NR | NR | NR | MLP: 953 kFPS; 1,019 GOPs | 2.05 W (MLP) | NR | 8 |
| [8] (Cyclone V) | 20,431 | 17,047 | NR | 393 | 61.9 MHz (CV); 124.5 MHz (ZU+) | NR | NR | 1.62 mJ (ZU+); 3.32 mJ (CV) | Comparable to FP models (claimed) | 7.5 |
| [9] | 6% → 69% | 5% → 54% | 1% → 18% | 5% → 26% | NR | NR | NR | NR | NR | 9 (rubric) |
| [10] | NR | NR | ≈ 6× fewer than prior | ≈ 9× more efficient | NR | NR | 3.07 FPS | NR | NR | 7 |
| [11] | 84.65% util. | NR | NR | NR | 100 MHz | NR | multi-FPS | ≈ 2.04 W (Vivado est.) | NR | 6 |

Table 1B note: [9] percentages are for 16 → 324 PEs on VC709 (433,200 LUTs available); percentages are rounded, so ratios derived from them are approximate.

Table 2. Quantisation comparison

| Ref | Original → target precision | Method | Bit width | Accuracy effect | Hardware effect | Limitation |
| --- | --- | --- | --- | --- | --- | --- |
| [1] | FP32 → INT8 | Symmetric per-tensor PTQ; INT32 accumulate; per-layer requant; INT4/6/8 study | 8 | Small loss reported (values NR) | Verilog INT8 MACs; XC7A35T 100 MHz | Sigmoid activations; utilisation NR |
| [2] | Double → fixed-point | Per-layer (W,I) by range coverage and sensitivity; round and saturate | 8 / 16 mixed | 96.40% vs 96.55% (2k subset) | DSP rose 14% → 18% (tool mapping) | Softmax float; HLS |
| [3] | FP32 → 16-bit fixed | Fixed-point library modules | 16 | −0.26% | Fewer LUT/FF than HLS baseline | Not INT8 |
| [16] | FP32 → INT8 | TFLite-style asymmetric PTQ, no retraining | 8 | “Marginal” loss | ResNet50: −94% PDP; +420% DSP eff.; +3100% LUT eff. | Device NR; large nets |
| [7] | FP → 1–2-bit | FINN QNN; 12 quantisation × folding designs | W1A1, W1A2, W2A2 | Accuracy per design NR here | 2.05 W for MLP | Not INT8 |
| [17] | FP → Q0.8 / Q1.7 | Two-layer CNN | 8 | 77% CIFAR-10 | 861 LUTs; 9 M9K; 1.2 W | Intel; venue unconfirmed |
| [18] | Float → fixed + approx. MAC | Shortened fixed-point; logic-based approx. units | Reduced | LeNet-5 MNIST | −78% DSP; −66% memory; ≈ −50% latency | HLS; device not in abstract |
| [5] | FP → 4–8-bit / Po2 | Post-training weight-matrix decomposition | 4–8 / Po2 | −1.3% average vs 8-bit designs | 33% average latency gain | Preprint |

Table 3. Parallelism scaling and design-space exploration

| Ref | Parallelism axis | Configurations / search space | Metrics reported | Result | Device | What is missing |
| --- | --- | --- | --- | --- | --- | --- |
| [3] | PM, PN, k, PFO per layer | Search under DSP and memory limits | Cycles, DSP, latency, FPS; comparison of LUT/FF vs cited works | 248 µs; layer-1 limits PM ≤ 1, PN ≤ 6 | Zynq-7020 | Power; sweep of overall parallelism; LUT/FF table not opened |
| [4] | Inner-parallel computation | CNN1 → CNN3 (also BCNN) | Latency, power | −60.32% latency, < +2% power (Arty-7); −44.93% / −45.85% vs non-optimised (CNN / BCNN) | Arty-7; VC709 | Image task; precision; resources |
| [7] | Folding × quantisation | 12 accelerators | Resources, throughput, power | MLP 953 kFPS at 2.05 W | PYNQ-Z1 | ≥ 3-bit; RTL control |
| [9] | Number of int8 PEs | 16, 64, 256, 324 | LUT, FF, BRAM, DSP % | LUT 6% → 69%; DSP 1% → 18% | VC709 | Power; small devices |
| [2] | HLS unroll / pipeline / dataflow | Off vs on | Latency, resources | 9.19 → 5.35 ms (1.71×); conv 2.14×; FC1 1.00× | XC7A200T | More than two points; power |
| [5] | Array configuration × bit width | Pareto search with analytical models | Accuracy, LUT, BRAM, FF, Fmax, latency, GOPS, est. power / energy | Latency −33% at −1.3% accuracy (avg.) | Arty A7-100T | Designs fill the device (LUT 95–99%) |
| [13] | Loop tiling, interchange, unrolling | Design space of loop optimisations | Performance vs hardware cost | Guides architecture for large CNNs | NR | Small networks; power |
| [20] | vec_fac, pe_num, reuse_fac | Ordered exploration | Latency vs DSP and bandwidth | Run-time-flexible OpenCL design | Intel Arria 10 | Non-Xilinx; not RTL |
| [14] | Tiling, unrolling (roofline) | Analytical design space | Throughput vs bandwidth | Foundational | Virtex-7 485T | Small devices; power |

Table 4. Memory architecture and dataflow

| Ref | Dataflow | On-chip memory strategy | Reuse focus | Reported effect | Device | Limitation |
| --- | --- | --- | --- | --- | --- | --- |
| [8] | Weight-stationary | Line buffers + feature-map buffers; fully BRAM-resident (FC replaced by conv) | Weights | Deterministic timing; low BRAM pressure claimed | Cyclone V; ZU+ | Not Artix-7; OCR task |
| [1] | Row-stationary | Line buffers, weight tiles, partial-sum BRAMs; BRAM-vs-cycles figure | Rows / windows | Zero RTL mismatches; details NR | XC7A35T | Only one dataflow retrieved |
| [11] | Streaming | Line-buffered convolution; AXI-Stream | Windows | LUT/routing-limited at 84.65% | Zynq-7020 | Routing limits scaling |
| [10] | Channel-parallel | Scoreboard-scheduled DRAM requests; BRAM | Off-chip traffic | ≈ 6× fewer DSP; ≈ 9× BRAM efficiency | XC7Z020 | DRAM dependent |
| [12] | Adaptive vs tiling | Per-layer on-chip buffers; TDM engine | Off-chip traffic | 1.29× speed; 23.7% dataflow compression | Cyclone V SoC | Intel device; lightweight CNNs, precision NR |
| [3] | Layer-pipelined | Interleaved weight memories; memory-volume model | Weights / feature maps | All data on chip for LeNet-5 | Zynq-7020 | Interleaving leaves memories under-filled |
| [30] | Improved row-stationary | PE line buffers; open-source VHDL | Rows | Improves PE utilisation for high output-channel layers | NR | FPGA target unverified |
| [15] | Loop / dataflow optimisation | Analytical | Loops | Foundational | NR | Seen via reference only |

Table 5. Power and energy methodology

| Ref | What is reported | Method / tool | Values | Weakness |
| --- | --- | --- | --- | --- |
| [8] | Energy per inference | Measured / tool NR here | 1.62 mJ (ZU+); 3.32 mJ (Cyclone V) | Method not retrieved |
| [4] | Power vs latency; GPU comparison | Measured (method NR) | < +2% power for −60% latency; −86.91% vs GPU | Absolute values NR |
| [5] | Estimated power and energy | Tool estimate | 1,097 mW; 33.77 µJ (8-bit SA) | Estimate; device-filling designs |
| [17] | Power vs CPU/GPU | Quartus Power Analyzer, psutil, nvidia-smi | 1.2 W | Intel; venue unconfirmed |
| [11] | Total on-chip power | Vivado post-implementation estimate | ≈ 2.04 W | Estimate; large PL power for Zynq |
| [21] | Power vs area | Vivado power; RTL clock gating | 3.644 → 2.442 W; LUT 44,392 → 102,580 | Device and precision NR |
| [26] (SNN, method reference) | Energy per inference | Vectorless vs SAIF | 0.053 mJ vs 0.028 mJ | Not a CNN |
| [2] | Not reported | — | — | No power or energy |

Table 5 note: the SNN row is included only to show the sensitivity of energy to estimation mode.

Table 6. Custom RTL versus HLS evidence

| Ref | Comparison | Finding | Relevance to project |
| --- | --- | --- | --- |
| [22] | FINN matrix-vector unit: RTL vs HLS | RTL smaller for small circuits; HLS more FF/BRAM; RTL shorter critical path; HLS ≥ 10× longer synthesis; LUT gap ≤ ~15% for large designs | Supports RTL for a small design |
| [23] | hls4ml vs FINN (MLPerf Tiny, Pynq-Z2) | hls4ml IC model used 58% fewer BRAMs but had 18.2× latency | Flows encode different resource–latency choices |
| [24] | HLS vs hand-mapped Verilog | HLS from better to up to 2× worse | Quality is design-dependent |
| [3] | Hand-written Verilog vs HLS LeNet-5 on Zynq-7020 | Fewer LUTs and FFs; higher FPS; lower latency | Closest like-for-like evidence |
| [2] | Vitis HLS on Artix-7 | Estimates drift; limited low-level timing control; 187 FPS | Motivates RTL timing control |
| [25] | HLS-generated variants | Many architectures from constraints alone; hand-written RTL variants expensive | Argues for parameterised RTL |
| [33] | HLS LeNet/CIFAR on Zynq-7000 | Early HLS estimates exceeded LUT capacity (120% and 166%) before optimisation | Illustrates HLS over-utilisation risk |

Table 7. Verification practice

| Ref | Golden model | Comparison level | Tolerance / result | Coverage | Across parallelism configs? |
| --- | --- | --- | --- | --- | --- |
| [1] | Bit-accurate Python INT8 reference | Logits, pre-sigmoid, intermediate accumulator and rescale nodes | Within one LSB; zero mismatches | Large test sets | Not shown (single design) |
| [2] | Matlab fixed-point reference | Conv output, FC/logits checkpoints; C sim and C/RTL co-sim | Acc(Matlab-FXP) = Acc(RTL) | 2,000-image subset; full 10k accuracy also reported | Not applicable (HLS directive on/off only) |
| [3], [4], [6], [7], [8] | NR in retrieved text | NR | NR | NR | NR |

Table 8. Similarity-score breakdown (rubric of Section 1.3)

| Ref | FPGA CNN (2) | Low-cost / Xilinx (2) | INT8 / fixed (2) | Custom RTL (1) | PE / MAC (1) | Resource / perf (1) | Multi-config / DSE (1) | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [3] Mao | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 9 |
| [5] Hernández Morales | 2 | 2 | 2 | 0 | 1 | 1 | 1 | 9 |
| [9] Flexible accelerator | 2 | 1 | 2 | 1 | 1 | 1 | 1 | 9 |
| [1] Shanker & Teo | 2 | 2 | 2 | 1 | 0 | 1 | 0.5 | 8.5 |
| [7] Tasci | 2 | 2 | 1 | 0 | 1 | 1 | 1 | 8 |
| [8] LPR | 2 | 1 | 2 | 1 | 0.5 | 1 | 0 | 7.5 |
| [4] Lee | 2 | 2 | 0 | 0 | 1 | 1 | 1 | 7 |
| [6] Ghani | 2 | 2 | 2 | 0 | 0.5 | 0.5 | 0 | 7 |
| [10] Low-cost edge | 2 | 2 | 1 | 0 | 1 | 1 | 0 | 7 |
| [18] Approx. MAC | 2 | 2 | 1 | 0 | 0.5 | 1 | 0.5 | 7 |
| [2] Cruz | 2 | 2 | 1 | 0 | 0 | 1 | 0.5 | 6.5 |
| [17] MaxNet | 2 | 1 | 2 | 0 | 0 | 1 | 0.5 | 6.5 |
| [11] Ship | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 6 |
| [20] Systolic-CNN | 2 | 0 | 0.5 | 0 | 1 | 1 | 1 | 5.5 |
| [16] Bel Haj Salah | 2 | 0 | 2 | 0 | 0 | 1 | 0 | 5 |
| [12] ADS-CNN | 2 | 1 | 0 | 0 | 0.5 | 1 | 0.5 | 5 |
| [13] Liu | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 5 |
| [21] Clock gating | 2 | 0 | 0 | 0 | 0 | 1 | 0.5 | 3.5 |

Table 8 note: high scores for [9] and [5] reflect methodological similarity; [9] uses a large Virtex-7 and [5] is a preprint, so they rank below [1] and [3] for architectural closeness.

Table 9. Papers that most directly challenge the project’s novelty

| Ref | Overlap with project | What it does not do |
| --- | --- | --- |
| [1] Shanker & Teo (2026) | Same device family, INT8 PTQ, Verilog MACs, line-buffered dataflow, bit-exact verification, LeNet-5 | No verified parallelism sweep or power; states educational rather than new-accelerator aim; lists DSE as future work |
| [3] Mao et al. (2023) | Hand-written Verilog, explicit parallelism knob, search, LeNet-5, low-cost Xilinx | 16-bit; no power; Zynq-7020 not Artix-7 |
| [5] Hernández Morales et al. (2026) | Artix-7, Pareto, LUT/BRAM/FF/Fmax/latency/power/energy for multiple bit widths | Preprint; estimated power; TinyML benchmarks; array fills device |
| [4] Lee et al. (2020) | Parallel optimisation with latency and power on Arty-7 | Non-image task; precision and resources NR |
| [7] Tasci et al. (2025) | Twelve configurations of folding and quantisation on 7-series | FINN/HLS; ≤ 2-bit; not RTL |

# 5  Critical cross-paper analysis and design implications

## 5.1  What the evidence collectively shows

Parallelism trades area for latency, but sub-linearly in area. In the VC709 PE-scaling study [9] the PE count grows about 20× (16 to 324) while LUT and FF utilisation grow only about 11× and BRAM about 5×; DSP utilisation rises from 1% to 18% (ratios derived from rounded percentages, so approximate). Fixed overheads (control, buffers, interconnect) are therefore amortised as parallelism grows, and the knee of the latency–area curve is device- and design-specific. On an Arty-7, Lee et al. report a 60% latency reduction for under 2% extra power [4], which suggests that on small devices static power dominates and dynamic power is not the main cost of adding parallelism — a hypothesis the proposed project can test directly.

Parallelism helps only the loops it is applied to. Cruz et al. obtained a 2.14× speed-up on the convolution stage (61% of cycles) but 1.00× on the FC1 stage (200,006 cycles, 37% of the total), giving 1.71× overall [2]. Mao et al. found that the first LeNet-5 layer, with one input and six output feature maps, caps parallelism at PM ≤ 1 and PN ≤ 6, so that layer becomes the pipeline bottleneck [3]. Together these results show that overall speed-up follows an Amdahl-type law, and that reporting only total latency hides which layer limits scaling.

Tool mapping can reverse expected resource trends. Moving from floating-point to fixed-point in [2] reduced BRAM from 31 to 4 blocks but increased DSPs from 34 to 43 and LUTs from 5,706 to 6,340 in post-synthesis results. The authors attribute the DSP increase to Vivado choosing DSP multipliers when the HLS schedule replicates MACs. In [18] the opposite design decision (implementing multiplications in logic) saved 78% of DSPs. Resource conclusions therefore depend on the multiplier-mapping choice as much as on arithmetic width.

Designs that fill the device confound cost-versus-precision curves. In the Arty A7-100T study [5], the 4-bit to 8-bit systolic arrays all occupy 97–99% of LUTs (62,531 vs 61,612 LUTs) and estimated power varies only between 1,060 and 1,166 mW, even though latency changes from 21.0 to 32.9 µs. When the array is sized to fill the device, lowering precision does not free area. A controlled study should fix the workload and vary the parallelism, leaving unused area visible.

## 5.2  Where sources disagree or use different conventions

- Accuracy versus parallelism. Sources that report both, such as [2], change accuracy only when arithmetic changes (for example float versus fixed), not when directives change parallelism. No source shows accuracy varying with parallelism at fixed arithmetic.

- RTL versus HLS. [22] finds RTL smaller for small designs, [24] finds HLS from better to 2× worse, and [3] finds hand-written Verilog smaller than HLS on LeNet-5. These are consistent if results depend on design size and structure, which favours a controlled RTL study on a small network.

- Precision of the word “INT8”. Sources mix symmetric per-tensor INT8 [1], per-layer mixed 8/16-bit fixed-point [2] and Q0.8/Q1.7 formats [17]. Claims about “INT8” are not directly comparable unless scale handling, accumulator width and rounding are stated.

## 5.3  Data-quality flags

- [2] reports 6,340 LUTs in the text and 6,324 in its comparison table (a small internal inconsistency); prefer the per-module tables when quoting numbers, and state which one was used.

- Several latency and resource values are Vitis HLS estimates, not post-implementation results [2]; power is often a tool estimate [5][11].

- [5] is an arXiv preprint and [9], [20], [22]–[25] are arXiv reports; their claims have not been confirmed against published versions in this survey.

- An Artix-7 depthwise-separable accelerator listed on ResearchGate reports an on-chip power of about 100 W; this is implausible for an Artix-7 and the source was excluded (Appendix B).

## 5.4  Quantitative sanity checks for the proposed design

The project deck does not fix the exact layer sizes. To reason about what a 1/4/8-MAC study can and cannot show, take an illustrative network: a 3×3 convolution with 8 filters on a 28×28 image (26×26×8 outputs), 2×2 max-pooling (13×13×8 = 1,352 values) and an FC layer 1,352→10. The MAC counts are 26 × 26 × 8 × 9 = 48,672 for convolution and 1,352 × 10 = 13,520 for FC, giving 62,192 MACs per image (illustrative). Assuming ideal issue of P MACs per cycle at 100 MHz, the compute time is:

Table 10. Ideal compute cycles and latency for the illustrative network (no pipeline fill, no memory stalls)

| MAC lanes P | Ideal cycles | Latency at 100 MHz | Ideal speed-up | DSP slices if one per lane (of 90) |
| --- | --- | --- | --- | --- |
| 1 | 62,192 | 621.9 µs | 1× | 1 (1%) |
| 4 | 15,548 | 155.5 µs | 4× | 4 (4%) |
| 8 | 7,774 | 77.7 µs | 8× | 8 (9%) |
| 16 | 3,887 | 38.9 µs | 16× | 16 (18%) |
| 32 | 1,944 | 19.4 µs | 32× | 32 (36%) |
| 64 | 972 | 9.7 µs | 64× | 64 (71%) |
| 90 | 692 | 6.9 µs | 90× | 90 (100%) |

Table 10 note (illustrative calculation, not from the literature): device DSP count is from the Artix-7 35T datasheet — verify. Real speed-up will be below P because of pipeline fill, control and memory limits.

Several design implications follow from this arithmetic.

- DSPs are not the limit at 1/4/8 lanes. Even 8 lanes use about 9% of 90 DSPs. Any resource ceiling observed on an XC7A35T at 1–8 lanes will come from LUTs, routing, BRAM banking or timing, as in [11]. A sweep that stops at 8 lanes may therefore show flat resource curves and miss the knee; extending to 16–32 lanes (or to a larger network) is needed to reach the device limit.

- Output-channel parallelism saturates early. With 8 filters and 10 classes, parallelism across output channels and neurons is capped near 8–10 lanes, mirroring the PN ≤ 6 limit in [3]. Going beyond that requires kernel-window parallelism (9 MACs per 3×3 window) or spatial parallelism, which have very different memory-bandwidth demands.

- Memory mapping depends on the parallelism axis (analysis). Output-channel parallelism needs P weights per cycle but only one shared activation; these P INT8 weights can be packed in one wide BRAM word. Window or spatial parallelism needs several activations per cycle from line-buffer taps. The choice of axis therefore changes both BRAM banking and LUT cost.

- Storage, not arithmetic, is the first hard limit. The illustrative network needs about 13.6 KB of INT8 weights (13,592 bytes), which is only a few BRAM36 blocks. By contrast, the network of [2] has about 202,620 weights (1,620 + 200,000 + 1,000), i.e. about 198 KiB at INT8, which is roughly 88–99% of the 1,800 Kb BRAM (datasheet) before any activation storage. That network does not fit an XC7A35T alongside its feature maps.

- Measurement definition can dominate the result. Loading one 784-byte image over a 115,200-baud UART takes about 68 ms (784 × 10 bits ÷ 115,200), roughly 875 times the ideal 8-lane compute time of 77.7 µs. Reported latency must therefore be measured on chip between start and done, and end-to-end host time should be reported separately.

## 5.5  Accuracy is invariant to parallelism — reframing the research question

The project’s research question lists accuracy alongside latency, throughput, resources and power as quantities that trade off with parallelism. If every configuration executes the same INT8 arithmetic, the classification results are identical by construction. The meaningful trade-off surface is latency–area–power–energy across parallelism. Accuracy enters only if precision, quantisation scheme or activation approximation is also varied (as in [1], [2] and [5]). The recommended plan (Section 10) therefore treats accuracy as a verified invariant for the parallelism sweep and as a genuine variable only on an optional precision axis.

# 6  Research gaps

Each gap is stated with the evidence that supports it, the limitation, its importance, existing attempts, the remaining unresolved point and the way the project can address it. Gaps are ordered by how defensible they are given the retrieved evidence.

## Gap 1 — Controlled MAC-lane scaling with a full metric set on an XC7A35T-class device

What existing research does. Demonstrates parallelism in several forms: per-layer allocation [3], inner-parallel optimisation with power on Arty-7 [4], FINN folding [7], PE-count scaling [9] and HLS directives [2].

Evidence. [3] reports latency and DSP counts but no power; [4] is a non-image task with precision NR; [7] is ≤ 2-bit; [9] uses a VC709; [2] compares two points.

Limitation. No verified source sweeps an integer MAC-lane parameter on a fixed workload and reports LUT, FF, DSP, BRAM, Fmax/WNS, latency, throughput, power and energy together on an XC7A35T-class device.

Why it matters. On a 90-DSP device the ceiling is likely set by LUT, routing and BRAM banking [11], so trends extrapolated from large devices [9] may not hold.

Existing attempts. [3], [4], [7], [9], [2].

Remaining gap. The combined experiment: fixed workload, at least five lane counts, small Artix-7, complete metrics.

How the project addresses it. A top-level parameter P in the Verilog RTL, with 1, 4, 8 as the core points and additional points (2, 16, and larger if the network permits) to expose the knee.

## Gap 2 — Credibility and disclosure of power and energy methodology

What existing research does. Reports total power, energy per inference or power ratios [4][5][8][11][17].

Evidence. [2] reports no power or energy; [5] and [11] use tool estimates; [26] shows a near-2× change in energy per inference between vectorless and SAIF-based estimation.

Limitation. Power is rarely reported with its estimation mode, switching-activity source or confidence level, and rarely linked to the same design points as resources and latency.

Why it matters. Without a stated method, energy-efficiency conclusions cannot be reproduced or compared.

Existing attempts. [8] (energy per inference across two devices), [17] (multi-tool comparison), [21] (RTL power optimisation).

Remaining gap. Per-configuration post-route power with disclosed method, on the same devices as resource and timing results.

How the project addresses it. Report vectorless and SAIF-driven post-implementation power for every configuration, with the Vivado confidence level. If the Basys 3 has no on-board current sensing (check the reference manual), any external board-level measurement should be reported with its resolution and as an additional, coarse data point.

## Gap 3 — Explicit DSP-versus-LUT multiplier mapping under parallelism

What existing research does. Reports DSP and LUT use for fixed-point designs [2][16][18].

Evidence. [2] observed DSPs rising from 34 to 43 (14% to 18%) for fixed-point because of tool mapping; [18] deliberately moved multiplication into logic and saved 78% of DSPs.

Limitation. Multiplier mapping is treated as a tool side effect, not as a controlled variable.

Why it matters. On a 90-DSP device a few dozen lanes exhaust DSPs, and LUT-based multipliers change timing and power.

Existing attempts. [18] (approximate logic MACs) and [2] (observation).

Remaining gap. A controlled comparison of DSP-mapped and LUT-mapped INT8 MACs across lane counts.

How the project addresses it. Add a multiplier-mapping parameter (DSP or logic, controlled with synthesis attributes) as a second experimental factor.

## Gap 4 — BRAM banking and memory bandwidth versus parallelism

What existing research does. Uses on-chip buffers, line buffers and tiling [1][8][10][11][12].

Evidence. [3] notes that interleaved weight storage leaves memories under-filled as parallelism rises; [9] shows BRAM growing from 5% to 26% with PE count; [11] is routing/LUT-limited.

Limitation. Memory organisation is described but its cost as a function of parallelism is not isolated.

Why it matters. A 50-BRAM device can run out of usable ports or banks before it runs out of DSPs.

Existing attempts. [3] (memory-volume model), [10] (bandwidth scheduling).

Remaining gap. Quantified BRAM/LUTRAM banking cost and its effect on Fmax and latency at each lane count.

How the project addresses it. Pack P weights per BRAM word for output-channel parallelism, report BRAM/LUTRAM counts per configuration, and compare against a banked alternative.

## Gap 5 — Bit-exact equivalence across a family of configurations

What existing research does. Performs bit-exact or checkpoint verification for one design point [1][2].

Evidence. [1] reports zero mismatches within one LSB; [2] requires accuracy equality across Matlab and RTL stages.

Limitation. No verified source shows that every parallelism configuration yields identical logits.

Why it matters. Banking, accumulation order and pipeline flushing bugs appear only at particular lane counts.

Existing attempts. [1], [2].

Remaining gap. Automatic regression of all configurations against one integer golden model on the full test set.

How the project addresses it. Run the 10,000-image MNIST test set through every configuration and compare logits (not only class labels) with the Python integer reference; require zero mismatches.

## Gap 6 — Analytical model of scaling validated against measurement

What existing research does. Provides analytical models for large designs [13][14][20] and latency/resource models inside a Pareto framework [5]; [3] derives per-layer allocation formulas.

Evidence. The Amdahl-type behaviour observed in [2] and [3] is described but not turned into a validated predictive model for a small device.

Limitation. Prediction error is rarely reported.

Why it matters. A model that predicts cycles, LUT, DSP and BRAM from P lets designers select a configuration without exhaustive synthesis.

Existing attempts. [3], [5], [13], [14], [20].

Remaining gap. A simple, fitted model with reported error percentages for an XC7A35T design.

How the project addresses it. Fit cycles = ⌈MACs/P⌉ + overhead per layer, and resource models linear in P, then report the prediction error for held-out lane counts.

## Gap 7 — Reproducible, open evaluation flow for small-device DSE

What existing research does. Releases code and Vivado projects in some cases [1][2].

Evidence. Most other sources in the retrieved set report results without artifacts (NR).

Limitation. Sweeps are hard to reproduce without parameterised RTL and scripts.

Why it matters. Reproducibility is the difference between a demonstration and a reusable design methodology.

Existing attempts. [1] (training-to-RTL path), [2] (GitHub workflow), [25] (constraint-driven variants in HLS).

Remaining gap. A scripted, parameter-driven RTL sweep with reports and plots for a small Artix-7.

How the project addresses it. Publish RTL, testbenches, Python reference, Vivado Tcl scripts and result tables.

# 7  Research-gap matrix

Table 11. Research dimension, literature status and potential contribution

| Research dimension | Existing literature | Strongest paper | What they achieve | Limitation | Remaining gap | Potential contribution of the project |
| --- | --- | --- | --- | --- | --- | --- |
| CNN architecture | LeNet-5-class and single-conv MNIST networks dominate; MobileNet at low bit widths on larger parts [2][3][8][10] | [3] LeNet-5 Verilog; [2] MnistConv | Working accelerators at 248 µs [3] and 5.35 ms [2] | Network size rarely justified against a device resource model; FC layers dominate weights [2] | Network selected from a resource/latency model for XC7A35T | Justify the 2-layer CNN against BRAM and lane-count limits (Section 5.4) |
| Quantisation | INT8 PTQ [1][16], per-layer fixed-point [2], ≤ 2-bit FINN [7] | [1] symmetric per-tensor INT8 with bit-exact reference | Small accuracy loss; reproducible arithmetic contract | Hardware side effects (DSP/LUT mapping) not isolated | Link quantisation choice to multiplier mapping and resources | INT8 baseline plus multiplier-mapping factor; optional precision axis |
| FPGA platform | Zynq-7020, Arty-7, XC7A200T, VC709, Cyclone V [2][3][4][8][9] | [1] XC7A35T; [6] Basys3 | Feasibility on the target die | Few papers use the smallest Artix-7; resource values NR | Complete metrics on XC7A35T | Report full utilisation and timing on the Basys 3 die |
| RTL architecture | Verilog/VHDL designs [1][3][8][9] | [3] parameterised module library | Fewer LUT/FF than HLS on Zynq-7020 | 16-bit; no power; architecture not open on Artix-7 | Parameterised INT8 RTL with open code on Artix-7 | Open parameterised generator with P, mapping and dataflow controls |
| MAC parallelism | Per-layer [3], inner-parallel [4], folding [7], PE count [9] | [3] and [4] | 248 µs; −60% latency at < 2% power | Each covers part of the metric set or a different device | Lane sweep with full metrics on XC7A35T | Sweep P ∈ {1, 2, 4, 8, 16, …} with all metrics (Gap 1) |
| Memory | Line buffers, BRAM-resident designs, tiling [1][8][10][11][12] | [8] BRAM-resident weight-stationary | Deterministic on-chip execution | Banking cost versus parallelism not isolated | BRAM/LUTRAM banking cost per lane count | Report banking and its effect on Fmax (Gap 4) |
| Dataflow | WS [8], RS [1], adaptive [12], surveys [29][30] | [1] and [8] | Documented dataflow choices on small devices | No like-for-like WS/OS/RS comparison on Artix-7 | Measured dataflow comparison on one network | Optional WS-vs-OS variant of the same RTL |
| DSE | Analytical [13][14][20], search [3], Pareto [5] | [5] Pareto on Arty A7-100T; [3] search | Accuracy–latency Pareto; per-layer allocation | Preprint; device-filling designs; small networks absent | Reproducible small-device DSE with validated model | Fitted analytical model with reported error (Gap 6) |
| Power | Estimates and comparisons [4][5][8][11][17][21] | [8] energy per inference; [4] power vs latency | mJ-level energy on two devices; power ratios | Estimation mode rarely stated; some studies omit power [2] | Per-configuration power with disclosed method | Vectorless + SAIF power for every configuration (Gap 2) |
| Verification | Bit-exact [1]; staged co-simulation [2] | [1] | Zero mismatches within one LSB | Single design point | Equivalence across all parallelism configurations | Full 10k-image regression per configuration (Gap 5) |
| Resource–performance trade-off | Pareto [5], PE scaling [9], folding [7], power–area [21] | [5] | Pareto over accuracy and latency; power and energy estimates | Different devices and precisions; estimated power | Latency–area–energy surface on XC7A35T | Pareto plots from measured/implemented data with model overlay |

# 8  Novelty audit and the critical question

## 8.1  Contribution-by-contribution audit

| Candidate contribution | Verdict | Evidence |
| --- | --- | --- |
| A. Lightweight CNN on FPGA | Common | [2][3][6][8][18] and many others |
| B. INT8 CNN on FPGA | Common | [1][6][8][16][17] |
| C. Artix-7 implementation | Established; XC7A35T-class thinner | [1] (XC7A35T), [6] (Basys3), [2] (XC7A200T), [4], [5] |
| D. Custom Verilog CNN accelerator | Established | [1][3][9] |
| E. Parallel MAC architecture | Common | [3][9][11] |
| F. Multiple PE / parallelism configurations | Partly done | [3][4][7][9], but on other devices, precisions or task types |
| G. Resource–performance trade-off analysis | Done for other devices; thinner on XC7A35T | [5][9][2] |
| H. Power / energy analysis | Often weak | [2] none; [5][11] estimates; [26] shows sensitivity |
| I. Combining all of the above | Potentially underexplored but incremental unless rigour is added | No verified source combines all elements |

## 8.2  The critical question

If you implement a lightweight INT8 CNN in custom Verilog on an Artix-7 and compare multiple parallelism configurations using LUT, FF, DSP, BRAM, latency, throughput, timing, power and energy, what is your contribution?

Already well established. INT8 PTQ, Verilog MAC engines and LeNet-class demonstrations on Artix-7. A 2026 paper already presents nearly the same training-to-RTL pipeline on the same die family with bit-exact verification [1]. Parallelism trends (more lanes, less latency, more area) are also well known [3][4][9].

Incremental. A 1/4/8 MAC sweep with the standard metrics. On its own it repeats existing results on a different device.

Potentially underexplored. The rigorous combination on an XC7A35T: controlled lane scaling, explicit multiplier-mapping and banking factors, disclosed power methodology (vectorless and SAIF), a validated analytical model with reported error, per-configuration bit-exact equivalence, and open artifacts.

What must be added to be research-worthy. (i) A wider sweep with at least five lane counts and per-layer allocation. (ii) A second factor (multiplier mapping, and optionally dataflow or precision). (iii) A predictive model checked on held-out configurations. (iv) Power with stated method. (v) Full-test-set logit equivalence for every configuration. (vi) Released artifacts.

Methodological caution. Because accuracy is invariant to parallelism at fixed INT8 arithmetic (Section 5.5), do not claim an accuracy–parallelism trade-off. Frame the trade-off as latency–area–power–energy, with accuracy as a verified invariant.

# 9  Final research-gap statements

### Version 1 — Simple (for explaining to the guide)

Many papers run small neural networks on FPGAs, and many use 8-bit numbers. Very few show, on the same tiny Artix-7 chip, how adding more parallel multipliers changes speed, chip area, timing and power together, while also proving that the hardware produces exactly the same answers as the software. That is the gap we address.

### Version 2 — Technical (for the project presentation)

Prior work studies parallelism, quantisation and power separately, mostly on larger or different devices, and often reports estimated or no power. We build a parameterised Verilog INT8 accelerator and sweep MAC lanes on an XC7A35T, reporting LUT, FF, DSP, BRAM, Fmax, latency and post-route energy per inference with a disclosed power method. We validate an analytical scaling model against measurements and check bit-exact equivalence to a Python integer reference for every configuration.

### Version 3 — Academic (basis for a paper introduction)

FPGA CNN accelerators are well studied at scale, and quantised LeNet-class designs on Artix-7 have been reported [1][2][6]. However, the resource–latency–power trade-off of integer-lane parallelism on the smallest Artix-7 devices lacks a controlled, reproducible study. In the existing literature, power is frequently omitted [2] or estimated [5][11] without disclosure of switching-activity source, whose choice alone can change energy per inference by nearly a factor of two [26]; multiplier-to-DSP/LUT mapping is left to the toolchain [2][18]; and bit-exact verification is demonstrated for individual design points [1] rather than across a family of parallelism configurations. We address these three points with a parameterised RTL accelerator and an open, scripted evaluation flow.

# 10  Recommended contribution and experimental design

## 10.1  Contribution statement

- A parameterised, open Verilog INT8 accelerator for MNIST-class CNNs on the Artix-7 XC7A35T with parallelism, multiplier mapping and (optionally) dataflow selected by parameters.

- A complete latency–area–power–energy characterisation, with power method disclosed and a fitted analytical model validated on held-out configurations.

- A per-configuration bit-exact verification regime on the full test set.

## 10.2  Experimental factors

| Factor | Levels | Purpose |
| --- | --- | --- |
| MAC lanes P | 1, 2, 4, 8 (core: 1/4/8 from the project plan); extend to 16, 32 (and 64 if the network allows) | Expose the knee and the device ceiling |
| Multiplier mapping | DSP-mapped vs logic-mapped INT8 multipliers | Gap 3 |
| Parallelism axis | Output-channel vs kernel-window (where feasible) | Show memory-bandwidth dependence |
| Clock target | 50, 100, 150 MHz (as timing allows) | Energy versus frequency; timing closure |
| Dataflow (optional) | Weight-stationary vs output-stationary | Fill the dataflow gap |
| Precision (optional) | INT8 baseline; INT6/INT4 study | The only axis on which accuracy varies |

## 10.3  Metrics and measurement protocol

| Metric | How to obtain it | Reporting rule |
| --- | --- | --- |
| LUT, FF, DSP, BRAM (and LUTRAM) | Post-implementation utilisation report | Report post-implementation, not only post-synthesis |
| Fmax / WNS | Post-route timing report at each clock target | State constraint and slack |
| Latency | Cycle counter from start to done on chip | Report cycles and µs; separate end-to-end host time (UART loading dominates otherwise, Section 5.4) |
| Throughput | Back-to-back inference rate | Images per second and MAC/s |
| Power | Post-route report_power with SAIF from post-implementation simulation; vectorless as comparison | Disclose method and Vivado confidence level for every number |
| Energy per inference | Power × latency (dynamic and total, separately) | State which power is used |
| Accuracy / equivalence | Full 10,000-image logits vs Python integer model | Zero mismatches required for every configuration |
| Model quality | Fitted cycles and resource models | Report prediction error on held-out P |

## 10.4  Testable hypotheses

- H1: Latency falls roughly as ⌈MACs/P⌉ plus a small per-layer overhead until the layer with the smallest parallelisable dimension becomes the bottleneck.

- H2: LUT and FF overheads grow more slowly than P (fixed control amortised), while DSP use grows linearly with P for DSP-mapped multipliers.

- H3: On this device, total power varies little with P compared with latency, so energy per inference decreases with P until routing or frequency effects intervene.

- H4: DSP-mapped and logic-mapped multipliers differ measurably in Fmax and power, and the crossover lane count depends on the DSP budget.

These are hypotheses to test, not results; any of them may be refuted by measurement, which is itself a publishable outcome.

## 10.5  Threats to validity

- Single workload. MNIST may saturate early; consider a second small workload if time permits.

- Tool dependence. Results depend on the Vivado version and strategy; record both.

- Power estimation. Post-route estimates are still models; state confidence and, where possible, compare with an external measurement.

- PTQ calibration. Report the calibration set and scale computation so the INT8 contract is reproducible.

# 11  Limitations of this survey

- Most IEEE Xplore and ACM full texts could not be opened, so IEEE-venue papers are under-represented; the search strings in Appendix A should be re-run there before novelty claims are made.

- Three papers were read in substantial part [1][2][3]. Others were analysed from abstracts, excerpts and tables retrieved by search; values in figures and unopened tables are NR.

- Some sources are preprints [5][9][20][22]–[25]; their content may change in published versions.

- Device resource figures for the XC7A35T and XC7A200T come from datasheet knowledge and are not from the surveyed papers.

- Section 5.4 uses an illustrative network; recompute with the project’s actual layer sizes.

# References

[1] M. Shanker and T. H. Teo, “Integer Intelligence: A Reproducible Path from Training to FPGA,” Electronics, vol. 15, no. 5, Art. 1117, 2026. doi:10.3390/electronics15051117.

[2] C. Cruz, M. Robles and I. Bravo, “A numerical workflow for post-training fixed-point CNN deployment on FPGA,” Array, vol. 30, Art. 100885, Jul. 2026. doi:10.1016/j.array.2026.100885.

[3] N. Mao, H. Yang and Z. Huang, “A Parameterized Parallel Design Approach to Efficient Mapping of CNNs onto FPGA,” Electronics, vol. 12, no. 5, Art. 1106, 2023. doi:10.3390/electronics12051106.

[4] J. Lee, J. He and K. Wang, “FPGA-based neural network accelerators for millimeter-wave radio-over-fiber systems,” Opt. Express, vol. 28, no. 9, p. 13384, 2020 (arXiv:2002.08205).

[5] J. J. Hernández Morales, G. Mentzos, F. Hannig, K. Balaskas, G. Zervakis, J. Henkel and J. Teich, “Co-Design of CNN Accelerators for TinyML using Approximate Matrix Decomposition,” arXiv:2604.16113, Apr. 2026 (preprint).

[6] A. Ghani, A. Aina and C. H. See, “An Optimised CNN Hardware Accelerator Applicable to IoT End Nodes for Disruptive Healthcare,” IoT, vol. 5, no. 4, pp. 901–921, 2024. doi:10.3390/iot5040041.

[7] M. Tasci, A. Istanbullu, V. Tumen and S. Kosunalp, “FPGA-QNN: Quantized neural network hardware acceleration on FPGAs,” Appl. Sci., vol. 15, no. 2, Art. 688, 2025. doi:10.3390/app15020688.

[8] “A resource-constrained CNN accelerator for real-time license plate character recognition on FPGA platforms,” Integration, the VLSI Journal, 2026 (authors not retrieved). doi:10.1016/j.vlsi.2026.102654.

[9] “A Flexible FPGA Accelerator for Convolutional Neural Networks,” arXiv:1912.07284 (authors not retrieved).

[10] “An Efficient CNN Accelerator for Low-Cost Edge Systems,” ACM, 2022 (authors and journal name to be confirmed). doi:10.1145/3539224.

[11] “FPGA-Based CNN Acceleration on Zynq-7020 for Embedded Ship Recognition in Unmanned Surface Vehicles,” Sensors, vol. 26, no. 5, Art. 1626, 2026 (authors not retrieved). doi:10.3390/s26051626.

[12] Y. Wan, X. Xie, J. Chen, K. Xie, D. Yi, Y. Lu and K. Gai, “ADS-CNN: Adaptive dataflow scheduling for lightweight CNN accelerator on FPGAs,” Future Gener. Comput. Syst., vol. 158, pp. 138–149, Sep. 2024. doi:10.1016/j.future.2024.04.038.

[13] Y. Liu, Y. Ma, B. Zhang, L. Liu, J. Wang and S. Tang, “Improving the computational efficiency and flexibility of FPGA-based CNN accelerator through loop optimization,” Microelectron. J., vol. 147, Art. 106197, May 2024. doi:10.1016/j.mejo.2024.106197.

[14] C. Zhang, P. Li, G. Sun, Y. Guan, B. Xiao and J. Cong, “Optimizing FPGA-based accelerator design for deep convolutional neural networks,” in Proc. ACM/SIGDA ISFPGA, 2015, pp. 161–170.

[15] Y. Ma, Y. Cao, S. Vrudhula and J.-S. Seo, “Optimizing loop operation and dataflow in FPGA acceleration of deep convolutional neural networks,” in Proc. ACM/SIGDA ISFPGA, 2017, pp. 45–54 (seen via reference list of [3]).

[16] O. Bel Haj Salah, S. Messaoud, M. A. Hajjaji, M. Atri and N. Liouane, “Post-training quantization for efficient FPGA-based neural network acceleration,” Integration, the VLSI Journal, vol. 105, Art. 102508, Nov. 2025.

[17] “FPGA-Based Accelerator for Quantized CNNs: High-Throughput Edge Deployment with Optimized Resource Utilization” (MaxNet), journalssystem.com (authors and venue not confirmed).

[18] “FPGA-Based Convolutional Neural Network Accelerator with Resource-Optimized Approximate Multiply-Accumulate Unit,” Electronics, vol. 10, no. 22, Art. 2859, 2021 (authors not retrieved). doi:10.3390/electronics10222859.

[19] P. A. Puttaswamy et al., “Resource optimization of approximate convolutional neural network-based …,” Bull. Electr. Eng. Inform. (year and volume not retrieved; first author as shown in page footer).

[20] “Systolic-CNN: An OpenCL-defined scalable run-time-flexible FPGA accelerator architecture for accelerating CNN inference in cloud/edge computing,” arXiv:2012.03177 (authors not retrieved).

[21] “RTL-Level Power Optimization of CNN Accelerators via Clock Gating and Sparsity-Aware MAC Suppression on FPGA,” Electronics, vol. 15, no. 11, Art. 2492, 2026 (authors not retrieved). doi:10.3390/electronics15112492.

[22] “On the RTL implementation of FINN matrix vector compute unit,” arXiv:2201.11409 (authors not retrieved).

[23] “Open-source FPGA-ML codesign for the MLPerf Tiny Benchmark,” arXiv:2206.11791 (authors not retrieved).

[24] “hls4ml: A Flexible, Open-Source Platform for Deep Learning Acceleration on Reconfigurable Hardware,” arXiv:2512.01463 (authors not retrieved).

[25] “FPGA-Based CNN Inference Accelerator Synthesized from Multi-Threaded C Software,” arXiv:1807.10695 (authors not retrieved).

[26] “A Time-Multiplexed Spiking Neural Network Accelerator with Pipelined Readout for FPGA Inference,” arXiv:2608.00595 (SNN; methodological reference only; authors not retrieved).

[27] “Efficient CNN Accelerator Based on Low-End FPGA with Optimized Depthwise Separable Convolutions and Squeeze-and-Excite Modules,” AI (MDPI), vol. 6, no. 10, Art. 244, 2025 (authors not retrieved).

[28] Z. Zhao, Y. Chen, P. Feng, J. Li, G. Chen, R. Shen and H. Lu, “A High-Throughput FPGA Accelerator for Lightweight CNNs With Balanced Dataflow,” arXiv:2407.19449.

[29] “Dataflow & Tiling Strategies in Edge-AI FPGA Accelerators,” arXiv:2505.08992 (survey; authors not retrieved).

[30] “Enhanced Accelerator Design for Efficient CNN Processing with Improved Row-Stationary Dataflow,” ACM, doi:10.1145/3649476.3658737 (authors and venue not retrieved).

[31] Y. LeCun, L. Bottou, Y. Bengio and P. Haffner, “Gradient-based learning applied to document recognition,” Proc. IEEE, vol. 86, no. 11, pp. 2278–2324, 1998 (seen via reference list of [3]).

[32] “Fast convolutional neural networks on FPGAs with hls4ml,” arXiv:2101.05108 (SVHN case study; authors not retrieved).

[33] “Acceleration and Energy Optimization of Convolutional Neural Networks on Zynq-7000 SoC: Comparative Analysis with GPU Platforms,” J. Eur. Syst. Autom., vol. 59, no. 1. doi:10.18280/jesa.590115 (authors not retrieved).

[34] “Real Time FPGA Based CNNs for Detection, Classification, and Tracking in Autonomous Systems: State of the Art Designs and Optimizations,” arXiv:2509.04153 (survey).

[35] F. Yan, A. Koch and O. Sinnen, “A survey on FPGA-based accelerator for ML models,” arXiv:2412.15666.

[36] “Architectural Design and Performance Analysis of FPGA based AI Accelerators: A Comprehensive Review,” arXiv:2603.08740 (survey).

## Foundational works cited from bibliographic memory (not retrieved in this session — verify before citing)

- Y.-H. Chen, T. Krishna, J. Emer and V. Sze, “Eyeriss: An energy-efficient reconfigurable accelerator for deep convolutional neural networks,” IEEE J. Solid-State Circuits, vol. 52, no. 1, 2017.

- N. P. Jouppi et al., “In-datacenter performance analysis of a Tensor Processing Unit,” Proc. ISCA, 2017.

- B. Jacob et al., “Quantization and training of neural networks for efficient integer-arithmetic-only inference,” Proc. CVPR, 2018.

- Y. Umuroglu et al., “FINN: A framework for fast, scalable binarized neural network inference,” Proc. ISFPGA, 2017.

- J. Duarte et al., “Fast inference of deep neural networks in FPGAs for particle physics,” J. Instrum., vol. 13, P07027, 2018.

## Cited in the project deck but not verified in this survey

- R. Foroumandi et al., “High-efficiency FPGA-based CNN accelerator with optimized data handling for convolution and fully connected layers,” IEEE Access, early access, 2025 — only an abstract fragment was seen; not used for quantitative comparison.

# Appendix A — Search strategy

- Exact project similarity: “INT8 CNN accelerator FPGA Artix-7”; “Verilog CNN accelerator Artix-7 Basys 3 MNIST”; “low cost FPGA CNN accelerator INT8”.

- Architecture: “FPGA CNN accelerator parallel MAC processing elements”; “FPGA convolution accelerator PE array Verilog”.

- Parallelism and DSE: “FPGA CNN accelerator PE parallelism design space exploration”; “number of processing elements scaling LUT DSP BRAM latency power”.

- Memory and dataflow: “FPGA CNN accelerator BRAM line buffer dataflow”; “weight stationary vs output stationary FPGA CNN”.

- Power: “FPGA CNN accelerator power energy resource utilization”.

- RTL and HLS: “hand-written RTL versus HLS CNN accelerator FPGA”; “Verilog RTL CNN accelerator FPGA”.

- Verification: “bit-exact verification CNN accelerator RTL Python golden model”.

- Citation chasing: reference lists of [2] and [3]; papers citing [3] and [14].

# Appendix B — Sources excluded or handled with caution

- Open-source repositories (for example bit-exact MNIST MLP or INT8 CNN projects on GitHub): not peer-reviewed; used only to confirm that bit-exact practice exists, not as evidence.

- An Artix-7 depthwise-separable accelerator listing about 100 W on-chip power (ResearchGate): implausible; excluded.

- Task-parallel OpenCL work on Intel devices (Gadea-Gironés et al., Microprocess. Microsyst. 98, 2023): energy-focused but Intel/OpenCL and dense-layer oriented; low similarity.

- Large-device, large-network accelerators (VGG/YOLO class): background only.

- Secondary-citation leads to verify: an Artix XC7A100T LeNet variant reported at 14,000 FPS and 98% accuracy, and a Zynq-7020 Int14 lightweight design at 11,630 FPS (both listed in the comparison table of [2]); a binarised licence-plate CNN on an XC7A100T (cited in [8]). Retrieve the primary papers before use.